Appendix 1 - Model Permit

This Appendix contains a model permit for unloading petrol into storage at petrol stations and filling of vehicle petrol tanks – see [insert relevant para from introduction] of this note and paragraph 3.6 of the General Guidance Manual on Policy and Procedures.

Notes:

- text in the model permit written in italics is advice to regulators.

- text in the model permit in [square brackets] offers choice to regulators or indicates where information needs to be inserted from the application;

- text bracketed with asterisks (eg *Alarms shall be tested at least once a week*.) may be omitted by a regulator where the past performance of the plant gives the local authority sufficient reassurance about operator compliance - “earned recognition”;

- the model permit has been drafted for local authorities in England and Wales.  Regulators in Scotland and Northern Ireland will need to amend the legal heading and, where appropriate, references to ‘Council’;

- references to ‘installation’ will need to be substituted with ‘mobile plant’ in relevant cases, and other amendments made accordingly;

- the purpose of the activity description is to set down the main characteristics of the activity, including any directly associated activities, so it is clear to all concerned what is being authorised by the permit and therefore what changes would need further approval.  Regulators are advised to include a description of any key items of abatement and monitoring equipment the operator intends to use or is using;

- it should normally be sufficient for records relating to simplified permits to be kept for no more than [24] months.  Where, however, as a result of a ‘low risk’ rating, inspections are undertaken less often, regulators may want to specify a period which ensures the records are available at the next inspection.

[         ] COUNCIL

POLLUTION PREVENTION AND CONTROL ACT 1999

Environmental Permitting Regulations 2010 (as amended)

Permit ref. no:

Name and address of person (A) authorised to operate the installation or mobile plant: Registered number and office of company: (if appropriate)

Address of permitted installation (B)

The installation boundary and key items of equipment mentioned in permit conditions are shown on the plans attached to this permit.

Activity description [The purpose of the description is to set down the main characteristics of the activity, including any directly associated activities, so it is clear to all concerned what is being authorised and therefore what changes would need further approval. Authorities are advised to include a description of any key items of arrestment and monitoring equipment the operator intends to use or is using. E.G:- The unloading of petrol into stationary storage tanks at a service station. The stationary technical unit comprises the following: - Storage Tanks: - Tank 1, Volume xxm3, Tank 2, Volume xxm3 etc. Delivery and vapour return hoses, Vapour recovery system, and Tank vapour release point all as described in the site plan. ]

Conditions

The person (A) is authorised to operate the activity at the installation (B) subject to the following conditions.

Petrol Delivery

- Vapours displaced by the delivery of petrol into storage tanks shall be returned through a vapour-tight connection line to either the mobile container delivering the petrol or a container on the site.

- Petrol delivery shall only be carried out using the [Stage I] petrol vapour recovery system and deliveries shall only be made when the system is fully operational.

Motor Vehicle Refuelling

- Motor vehicle refuelling with petrol shall only take place when the [Stage II] petrol vapour recovery system is fully operational and operating in accordance with the requirements of Condition 4.

- The petrol vapour capture efficiency of the [Stage II] petrol vapour recovery system shall be equal to or greater than 85% but less than 115% as certified by the manufacturer in accordance with relevant European technical standards or type approval procedures.

- Where the recovered petrol vapour is transferred to a storage tank, the vapour/petrol ratio shall be equal to or greater than 0.95 but less than or equal to 1.05.

- Where an automatic monitoring system has been installed, the petrol vapour capture efficiency shall be tested and the results recorded at least once every three years by checking that the vapour/petrol ratio under simulated petrol flow conditions, or by any other appropriate methodology. Any such automatic monitoring system shall automatically detect faults in the proper functioning of the [Stage II] petrol vapour recovery system and in the automatic monitoring system itself, indicate faults to the operator and automatically stop the flow of petrol from any faulty dispenser if the fault is not rectified within seven days.

- Where automatic monitoring systems have not been installed the in-service petrol vapour capture efficiency of the [Stage II] petrol vapour recovery systems shall be tested the results recorded at least once a year by checking that the vapour/petrol ratio under simulated petrol flow conditions, or by any other appropriate methodology.

- Where automatic monitoring systems have not been installed a *weekly functionality check* shall be undertaken to verify the operation of the vapour recovery system.

- A sign, sticker or other notification shall be displayed on, or in the vicinity of, the petrol dispenser, informing consumers that a [Stage II] petrol vapour recovery system is in use.

Incident Reporting

- In the event of any incident at the site which could have an impact beyond the site boundary, the operator shall notify Council by telephone without delay on xxxxxxxxx

Management

- A copy of this permit shall be kept at the permitted installation.  All staff who should be aware of its content shall be told where it is kept.

- All relevant staff shall receive the necessary training and instruction to enable them to comply with the conditions of this permit.

- The operator shall notify the Council of any changes to the persons nominated in the application as the primary point of contact, and deputy.

- Maintenance and testing of vapour recovery systems shall be recorded.

- All records made in compliance with this permit shall be kept in a written or computer log book or by using some other systematic method, and shall be clear and legible.   If any entry is amended, a clear statement of the reason for doing so shall be included.  Unless otherwise stated in this permit, all records required to be taken shall be kept available for inspection for at least 4 years from the date of its being made.  A copy of the manufacturer’s instructions referred to in this permit shall be available for inspection on request.

Best available techniques

- The best available techniques shall be used to prevent or, where that is not practicable, reduce emissions from the installation in relation to any aspect of the operation of the installation which is not regulated by any other condition of this permit.

Process changes

- If the operator proposes to make a change in operation of the installation, he must, at least 14 days before making the change, notify the regulator in writing.  The notification must contain a description of the proposed change in operation.  It is not necessary to make such a notification if an application to vary this permit has been made and the application contains a description of the proposed change.  In this condition ‘change in operation’ means a change in the nature or functioning, or an extension, of the installation, which may have consequences for the environment.

Right to appeal

You have the right of appeal against this permit within 6 months of the date of the decision.  The Council can tell you how to appeal [or supply details with the permit].  You will normally be expected to pay your own expenses during an appeal.

You will be liable for prosecution if you fail to comply with the conditions of this permit.  If found guilty, the maximum penalty for each offence if prosecuted in a Magistrates Court is £50,000 and/or 6 months imprisonment.  In a Crown Court it is an unlimited fine and/or 5 years imprisonment.

Our enforcement of your permit will be in accordance with the Regulators’ Compliance Code

Appendix 2 - Application form

Application for a permit for unloading petrol into storage at petrol stations and filling of vehicle petrol tanks

Local Authority Pollution Prevention and Control

Pollution Prevention and Control Act, 1999

Environmental Permitting (England and Wales) Regulations 2010

Introduction

When to use this form

Use this form if you are applying for a permit to a Local Authority to operate an installation unloading petrol into storage at petrol stations and filling of vehicle petrol tanks.

The appropriate fee must be enclosed with the application to enable it to be processed further.  When complete, send the form and the fee and any additional information to:

[Insert local authority address]

If you need help and advice

We have made the application form as straightforward as possible, but please get in touch with us at the local authority address given above if you need any advice on how to set out the information we need.

For the purposes of Section G of the form, a relevant offence is any conviction for an offence relating to the environment or environmental regulation.

| For Local Authority use |  |  |
| --- | --- | --- |
| Application reference | Officer reference | Date received |

LAPPC application form - to be completed by the operator A The basics A1 Name and address of the installation Postcode: Telephone:. A2 Details of any existing environmental permit or consent (for waste operations, include planning permission for the site, plus established use certificates, a certificate of lawful existing use, or evidence why the General Permitted Development Order applies.) A3 Operator details (The ‘operator’ = the person who it is proposed will have control over the installation in accordance with the permit (if granted).) Name: Trading name, if different:. Registered office address: Principal office address, if different:. Company registration number:.

A4 Any holding company? Is the operator a subsidiary of a holding company within the meaning of section 1159 of the Companies Act 2006? If “yes” please fill in details of the ultimate holding company.  Yes  No Name: Trading name, if different:. Registered office address: Principal office address, if different:. Company registration number:. A5 Who can we contact about your application? It will help to have someone who we can contact directly with any questions about your application. The person you name should have the authority to act on behalf of the operator - This can be an agent or consultant. Name and position: Telephone: Email:.

B The installation B1 What activities are or will be carried on at the installation? Please include “directly associated activities” – this term is explained in Annex III in Part B of the general guidance manual. B2 Why is the application being made?  new installation  change to existing installation means it now needs a permit B3 Site maps – please provide: A location map with a red line round the boundary of the installation Document reference: A site plan or plans showing where all the relevant activities are on site: a) the location of the fill points b) the buildings/structures c) the location of the petrol dispensers d) the height and location of the vent pipes e) the number, capacity and location of all storage tanks Document reference: B4 Technical documentation – please provide: a) Copy of “type approval” certificate where a site operates Stage II activities

| C The details C1 Which of the following vapour monitoring systems will be in use? a) active system with automatic monitoring  (tick relevant) b) active system without automatic monitoring  C2 Do you have environmental management procedures and policy?  Yes  No If yes, please supply a copy. C3 Are there any sites of special scientific interest (SSSIs) or European protected sites nearer than 500m to the proposed installation?  Yes  No If ‘yes’, is the installation likely to have a significant effect on the special scientific interest or European protected sites?  Yes  No If ‘yes’, please write on a separate sheet or enclose a relevant document explaining what the implications are for the purposes of the Conservation (Natural Habitats etc) Regulations 1994 (see appendix 2 of Annex XVII of the general guidance manual) Document reference: C4 Will emissions from the activity potentially have significant environmental effects (including nuisance)?  Yes  No If ‘yes’, please list the potential significant local environmental effects (including nuisance) of the foreseeable emissions on a separate document. Document reference: If ‘yes’, please enclose a copy of any environmental impact assessment which has been carried out for the installation under planning legislation or for any other purpose. Document reference: C5 Is the proposed installation sited under living quarters?  Yes  No |  |
| --- | --- |
| D Anything else? Please tell us of anything else you would like us to take account of: Document reference: |  |

E Application fee You must enclose the relevant fee with your application. If your application is successful you will also have to pay an annual subsistence charge, so please say who you want invoices to be sent to. Name and position: Telephone: Email:.

F Protection of information F1 Any confidential or national security information in your application? If there is any information in your application you think should be kept off the public register for confidentiality or national security reasons, please say what and why. General guidance manual chapter 8 advises on what may be excluded. (Do not include any national security information in your application. Send it, plus the omitted information, to the Secretary of State or Welsh Ministers who will decide what, if anything, can be made public.) Document reference : F2 Please note: data protection The information you give will be used by the Council to process your application. It will be placed on the relevant public register and used to monitor compliance with the permit conditions. We may also use and or disclose any of the information you give us in order to: - consult with the public, public bodies and other organisations; - carry out statistical analysis, research and development on environmental issues; - provide public register information to enquirers; - make sure you keep to the conditions of your permit and deal with any matters relating to your permit; - investigate possible breaches of environmental law and take any resulting action; - prevent breaches of environmental law; - offer you documents or services relating to environmental matters; - respond to requests for information under the Freedom of Information Act 2000 and the Environmental Information Regulations 2004; (if the Data Protection Act allows) - assess customer service satisfaction and improve our service. We may pass on the information to agents/representatives who we ask to do any of these things on our behalf. F3 Please note: it is an offence to provide false information It is an offence under regulation 38 of the EP Regulations, for the purpose of obtaining a permit (for yourself or anyone else), to: - make a false statement which you know to be false or misleading in a material particular; - recklessly make a statement which is false or misleading in a material particular; - intentionally to make a false entry in any record required to be kept under any environmental permit condition; - with intent to deceive, to forge or use a document issued or required for any purpose under any environmental permit condition. If you make a false statement: - we may prosecute you; and - if you are convicted, you are liable to a fine or imprisonment (or both).

G Declarations A and B for signing, please These declarations should be signed by the person listed in answer to question A3. Where more than one person is identified as the operator, all parties should sign. Where a company or other body corporate is the operator, an authorised person should sign and provide evidence of authority from the board. Declaration A: I/We certify EITHER - As evidence of my/our competence to operate this installation in accordance with the EP Regulations, no offences have been committed in the previous five years relating to the environment or environmental regulation. OR - The following offences have been committed in the previous five years which may be relevant to my/our competence to operating this installation in accordance with the regulations: Signature: Name: Position: Date: Declaration B: I/We certify that the information in this application is correct. I/We apply for a permit in respect of the particulars described in this application (including the listed supporting documentation) I/we have supplied. (Please note that each individual operator must sign the declaration themselves, even if an agent is acting on their behalf.) Signature: Name: Position: Date: Signature: Name: Position: Date: Signature: Name: Position: Date:

listed in [     ] in Part 2 of Schedule 1 to the Environmental Permitting Regulations